RCB-6201   High-Performance Octa-Core IoT SoM — MediaTek MT8391

Features

- MT8391（Genio 720）octa-core 64-bit CPU

- Arm Mali-G57 MC2 GPU with Vulkan 1.1 support

- 10 TOPS NPU

- Up to 5K display resolution or dual-screen output

- 4K HEVC video encoding and decoding

- OSM Size-L, 45x45mm

Specifications

| Category | Item | Specification |
| --- | --- | --- |
| Processor | CPU | MTK MT8391 (Genio 720), 2x Arm Cortex-A78 @ 2.4-2.6GHz, 6x Arm Cortex-A55 @ 2.0GHz |
| Memory | LPDDR5 | 8GB onboard, optional expansion |
| Storage | eMMC | 64GB onboard, optional expansion |
| Display | Display | 2x eDP; 1x MIPI-DSI/LVDS (1x 4-lane MIPI-DSI shared with dual-channel LVDS) |
| Camera | Camera | 2x 4-lane MIPI CSI |
| High-Speed Interfaces | USB | 2x USB 3.0 (1x OTG, 1x HOST), 3x USB 2.0 |
|  | PCIe Express | 1x Gen2 x1 |
| Network Interfaces | Ethernet | 1x RGMII |
| Storage | SDIO | 2x SDIO (1x 4-bit, 1x 8-bit) |
| General Purpose I/O Interfaces | Connectivity | 2x I2C, 3x SPI, 3x PWM, 16x GPIO |
|  | UART | 4x UART (including 1x console port) |
| Power Supply | Input Voltage | DC 12V |
| Dimensions | Mechanical Dimensions | OSM Size-L 45x45mm |
| Environmental | Operating Temperature | 0℃ to 60℃ (0% to 90% RH, non-condensing) |
|  | Storage Temperature | -20℃ to 70℃ |
| Operating System |  | Android 15 / Ubuntu 22.04 |

Dimensions (OSM Size-L 45*45mm)

Ordering Information

Product Selection

| Model Number | LPDDR5 | eMMC | Operating Temperature |
| --- | --- | --- | --- |
| RCB-6201-L01 | 8GB | 64GB | 0°C to 60°C |